# D5. Identify environmental and social risks and prepare Environmental and Social Management Plans

# Consulting Services for the Preparation of Detailed Designs and Technical Specifications for Energy Efficiency Measures and Distributed Solar PV Systems for Public Buildings in Guyana.

LC-OECS COMMISSION-414663-CS-QCBS, LOT 3 GUYANA

Reference: S00289

“This report is produced for the OECS Commission as part of the Project Preparation Activities of the Caribbean Efficient and Green-Energy Buildings with funding from the World Bank.”

Barcelona, 30th September 2025

Table of Contents

Revisions	3

List of abbreviations	4

1	Introduction	5

2	Description of the Project	7

3	Environmental and Social Baseline	10

Regional Democratic Council (RDC)- Region 3	10

Guyana Revenue Authority (GRA) – Parika	12

Guyana Elections Commission (GECOM) – Head Office	14

4	Environmental and Social Management Plan	17

4.1	Implementation Arrangements and Reporting	31

4.2	Institutional Strengthening and Capacity Building	35

4.3	Grievance Redress Mechanism	36

4.3.1	Stakeholder Engagement	37

4.3.2	Cost	38

Annex	41

Grievance Redress Forms	41

List of Tables

Table 1: Project locations for EE and DPV Interventions in Guyana	5

Table 2: RDC region 3: Government Administrative Buildings for CEBEG Project	11

Table 10: Results of the Environmental and Social Screening of the government administrative buildings	17

Table 11: Risk Assessment Matrix	19

Table 12: Identification of impacts on the government administrative buildings	20

Table 13: Mitigation and Monitoring Arrangements for Government Administrative Buildings	27

Table 14: Training Plan	35

Table 15: Stakeholders’, Interests, and Needs	37

Table 16: Stakeholder Consultation Engagement Plan	38

Table 17: Cost Estimates for all four institutions	40

Table 18: Grievance Submission Form	41

Table 19: Grievance Management Registration and Monitoring Sheet	43

Table 20: Grievance Disclosure/Release Form (Result of Grievance Management)	44

List of Figures

Figure 1: Frontal view of RDC – Region 3	10

Figure 2: Location of RDC – Region 3	11

Figure 3: Frontal view of GRA - Parika	12

Figure 4: Location of GRA - Parika	13

Figure 5: Frontal view of main building of GECOM	14

Figure 6: Location of GECOM	15

Figure 34 Government Administrative Buildings Employment Categories	16

Figure 9: Organisational Structure of the Project Implementing Unit	33

# Revisions

| Document Version | Checking Date | Checked by | Comments |
| --- | --- | --- | --- |
| V 0.0 | 02/06/2025 | AIGUASOL | Formatting and alignment, content development, attention to quality, timeline |
| V0.1 | 12/08/2025 | AIGUASOL | Draft sent to client |
| V0.2 | 31/08/2025 | World Bank | ESMPs need to be more structured, concise, and consistent, with clear coverage of all sites and removal of redundant or inaccurate content. Screening checklists, risk assessments, and clear assignment of responsibilities (contractor/institution) must be included, along with monitoring and reporting arrangements. Site-specific issues such as bat presence at the GRA building and potential mold/asbestos must be assessed and addressed. Budgets are unrealistic and require review and refinement to align with actual mitigation needs. |
| V.03 | 9/09/2025 | World Bank | Review of Environmental and Social Mitigation measures, Contractors’ responsibilities, social aspects including Grievance Redress mechanism and Stakeholder Consultation Plan and costing justification |
| V.04 | 30/09/2025 | Aiguasol | Final submission |

# List of abbreviations

| Abbreviation | Meaning |
| --- | --- |
| BOQ | Bill of Quantities |
| CEGEB | Caribbean Efficient and Green Energy Buildings |
| DPV | Distributed Solar Photovoltaics |
| DRE | Distributed Renewable Energy |
| EE | Energy Efficiency |
| EEM | Energy Efficiency Measures |
| ESF | Environmental and Social Framework |
| ESCP | Environmental and Social Commitment Plan |
| ESMF | Environmental and Social Management Framework |
| ESMP | Environmental and Social Management Plan |
| EPA ESS | Environmental Protection Agency Environmental and Social Standards |
| GEA | Guyana Energy Agency |
| GECOM GHG | Guyana Elections Commission Greenhouse Gas Emissions |
| GLSC | Guyana Lands and Surveys Commission |
| GNBS | Guyana National Bureau of Standards |
| GPL | Guyana Power and Light Incorporated |
| GRA GWI | Guyana Revenue Authority Guyana Water Incorporated |
| GWP | Global-warming potential |
| HECI | Hinterland Electrification Company Incorporated |
| LCDS | Low Carbon Development Strategy |
| LMP NDC OECS | Labour Management Procedure Neighbourhood Democratic Council Organisation of Eastern Caribbean States |
| PDO | Project Development Objective |
| PPG | Project Preparation Grant |
| PV | Photovoltaics |
| RDC | Regional Democratic Council |
| RE | Renewable Energy |
| SOP | Series of Projects |
| WB | World Bank |
| WHO | World Health Organisation |

# Introduction

The Caribbean Efficient and Green Energy Buildings (CEGEB) Project will address shared challenges in the energy sector within the Caribbean. It will support investment in energy efficiency measures (EEMs) and distributed solar photovoltaics (DPV) systems. The energy efficiency (EE) retrofits will include passive and active measures along with improvements in building-control systems. Passive EE measures will incorporate energy-efficient windows, shading, wall and roof insulation, cool surfaces, etc. Active EE measures will include the replacement of existing inefficient building equipment and appliances, or installation of new energy efficient equipment and appliances, such as lighting systems, air-conditioning equipment, ceiling fans, and refrigerators or freezers using refrigerants with low or no global-warming potential (GWP), where appropriate. Improvements in control systems will include smart controls, sensors, and energy management systems to meet EE objectives. The Project will also support safe disposal of used equipment and materials. These works will be implemented at government buildings and facilities including central and municipal administrative buildings, universities and schools, hospitals and clinics, stadiums, and other publicly owned facilities. Locations are expected to be on government-owned land in urban and peri-urban settings in the countries.

Table 1: Project locations for EE and DPV Interventions in Guyana

| Sub-Project Site Type | Sub-Project Site Name | Locations | Regions |
| --- | --- | --- | --- |
| Government Administrative Buildings | Regional Democratic Council | Vreed – en – hoop | Region 3 |
|  | Guyana Revenue Authority | Parika |  |
|  | Guyana Elections Commission, Head Office | Georgetown | Region 4 |
| Educational Institutions | Good Hope Secondary School | East Coast Demerara |  |
|  | West Ruimveldt Primary School | Georgetown |  |
|  | Yarrowkabra Secondary School | Linden/ Soesdyke Highway |  |
|  | Zeeburg Secondary School | Zeeburg | Region 3 |
| Healthcare/storage Institutions | Parika Health Center | Parika |  |
|  | Rosignol Health Center | Rosignol | Region 5 |
|  | National Ophthalmology Hospital | Port Mourant |  |
|  | Port Mourant District Hospital | Port Mourant |  |
|  | Ministry of Health Central Supplies Unit | Water Street, Georgetown | Region 4 |
|  | Ministry of Health Materials Management Unit |  |  |

The objective of this assignment is to define the EEMs and DPVs and prepare the detailed design, technical specifications, and bill of quantities (BOQ) for the bidding documents for implementing the proposed EEMs and DPVs investments. The part includes identification of environmental and social risks associated with the planned implementation and preparation of Environmental and Social Management Plans (ESMPs) in accordance with the Environmental and Social Management Framework (ESMF) as well as other national environmental legislation and regulations in Guyana.

Scope of Environmental and Social Management Plan

The works under this part entail identification of environmental and social risks and preparation of an ESMP including:

- Identification of environmental and social risks associated with the implementation of the EEMs.

- Preparation of site specific ESMPs.

- Provision of support to the Guyana Energy Agency in public consultations on audit and ESMPs.

Methodology

The approach adopted were as per requirements of the World Bank and Guyana’s national legal framework. Initially, screening of the environmental and social setting and any potential environmental and social impacts and risks were assessed using the World Bank’s Environmental and Social Screening Questionnaire and the Guyana - Environmental Protection Agency’s Guidelines for Preparation of an Environment Assessment and Management Plan. Secondary data was collected based on scientific, engineering, environmental and economic parameters, professional judgement, and consultation with the relevant stakeholders.

Limitations - Interdependencies in Team Deliverables

The timely completion of this report was partially constrained by dependencies on contributions from other team members. Specifically, certain sections of Chapter 2 (Description of the EEMs and DPVs) required input or data from parallel reports that were still in progress. As a result, there were delays in accessing critical information needed to complete this report comprehensively and within the originally scheduled timeframe. This interdependence among team deliverables underscores the need for improved coordination and sequencing in future project timelines.

# Description of the Project

Description of the Energy Efficient Measures and Distributed Photovoltaic Systems to be installed and key features

The optimal energy efficiency scenario for the government administrative buildings includes the addition of all the following proposed energy efficiency measures:

- Addition of an external insulation layer on the roof

- Replacing the windows in conditioned rooms to achieve code compliance

- Ceiling mounted fans in rooms

- Ceiling mounted fans in conditioned rooms

- Replacing fluorescent tubes with LED lighting

- Installing occupancy detection sensors in bathrooms

- Smart switches to control the electricity consumption in appliances

- Nighttime air conditioning shutdown

Design and Technology

Addition of an external insulation layer on the roof

An insulated steel roof sandwich panel with a high-density polyurethane core, enclosed between two layers of pre-lacquered galvanized steel for enhanced thermal insulation is recommended for an added layer of external insulation.  The insulation layer should be approximately 160 mm of mineral wool or 80 mm of polyurethane, depending on the desired thermal conductivity.

The panel proposed for the external insulation layer is an insulated steel roof sandwich panel featuring a mineral wool core for thermal insulation, encased between two layers of pre-lacquered galvanized steel. It is available in 160 mm thickness, ensuring optimal sealing and reduced thermal losses.

Replacing the windows in conditioned rooms to achieve code compliance

To achieve code compliance, double-glazing with two panes of glass separated by an insulating air layer is proposed. Performance is further enhanced by thermal break frames, which incorporate a low-conductivity barrier between the interior and exterior sections of the frame, effectively reducing heat transfer. Together, these technologies significantly reduce external heat gain and minimize thermal bridging, contributing to a cooler and more energy-efficient indoor environment. The total glass façade area of the conditioned spaces, fitted with double-glazing windows featuring thermal break frames and a 12 mm air gap, is estimated at approximately 81 m². Replacing windows in these areas is a key component of renovation efforts aimed at enhancing the thermal performance of the building envelope.

Ceiling mounted fans in rooms

Installing fans in the upper part of non-air-conditioned rooms improves the thermal sensation for occupants with a minimal increase in energy consumption. This includes the installation of at least two ceiling fans per classroom with a total of 26 fans.

Ceiling mounted fans in conditioned rooms

Installing fans in the upper part of air-conditioned rooms improves the thermal sensation for occupants with a minimal increase in energy consumption. This includes the installation of at least two ceiling fans per room.

Replacement of current lighting for (Light-Emitting Diode) LED technology lighting

The replacement of current lighting includes the change of fluorescent tubes and mercury vapour lighting by LED lighting.

Installing occupancy detection sensors in bathrooms for lighting operation

This measure involves the installation of five occupancy sensors that automatically switch lighting on or off based on the presence of people in each bathroom with the goal to eliminate unnecessary electricity when bathrooms are unoccupied.

Smart switches to control the electricity consumption in appliances

Smart switches on equipment such as computers and fans, among others, are recommended to allow them to be turned off outside of working hours and prevent excessive energy consumption.

Nighttime Air Conditioning shutdown

Since A/C equipment consumes one-third of the total energy consumption, ensuring it is properly shut down after work ensures significant savings. The installation of signs reminding the public to turn off the system is proposed, highlighting the impact of unnecessary increase in energy consumption and carbon emissions.

DPV Components

The solar system will comprise the following components:

- PV modules

- PV structure

- PV arrays

- Inverters

- Transformer

- Interconnection, control and monitoring

Commissioning

After installing the EEMs and DPV systems, the Contractor will transfer the responsibilities to the Government of Guyana. It involves a systematic quality assurance process that ensures all building system and components are installed, tested and operate according to design intent, performance standards and user needs (Mavetech, 2022). This process certifies that the requirements were met, installation is complete, and the building complies with grid and safety requirements (IFC, 2015) and energy efficiency upgrades and mechanical/electrical installations are fully functional, safe and optimized. Ultimately, commissioning should ensure that occupants are comfortable and able to use systems easily and long-term operational savings and low-maintenance performance.

Implementation of ESMP

The ESMP will be implemented throughout the Project's development and entire lifecycle, outlining the potential environmental, health, safety, and social impacts, along with the corresponding mitigation measures. Its implementation ensures that all project activities comply with both international and national requirements under the Programme.

Before commencement of works, several approvals and clearances would be required. Building permits/No Objections to install EEMs and DPVs must be granted by local authorities within communities in proximity to site as well as the Ministry of Education. For rural areas where construction vehicles like heavy machines may obstruct traffic, permission to utilize access roads may be needed. This establishes respect through consent with local authorities especially in rural communities. Usually, environmental construction permits are required; however, the EPA noted that they are not necessary for this Project.

# Environmental and Social Baseline

The government administrative buildings included in this ESMP are Regional Democratic Council – RDC (Region 3), Guyana Revenue Authority (GRA) – Parika and Guyana Elections Commission (GECOM) Head Office. All buildings within this ESMP will implement energy efficient measures and distributed solar photovoltaics.

### Regional Democratic Council (RDC)- Region 3

The RDC – Region 3 is in Vreed-en-Hoop, Region 3 (Essequibo Islands – West Demerara). It is an administrative building that belongs to the Government of Guyana. The date of construction is unknown, but it has had numerous upgrades and expansions over the years. The total area of construction is 1 070 m2.

In 2024, the total energy consumption was estimated at 135 375 kWh with an estimated cost of 7 964 091 GYD. When it comes to the CO2 emissions, the total was 90 159 kgCO2. If compliant with building codes, the energy consumption of the building is expected to reduce by 36 980 kWh with the new average consumption estimated at 98 395 kWh. Concurrently, CO2 emissions are also decreased by 24 622 kgCO2.

Currently, there are no renewable energy system installations in the building.

Figure 1: Frontal view of RDC – Region 3

Figure 2: Location of RDC – Region 3

Surrounding the RDC – Region 3 are mainly clusters of small to medium size businesses located on the main access road, that provide goods and services. The Vreed-en-Hoop Market and Stelling are nearby and along the Demerara River. There is also the West Demerara Shore base Facility within proximity to the Office.  The RDC office is situated in a socially active, economic hub, where administrative functions, community aspects, local commerce, and river transport converge, which makes it a central point in the everyday life of the region. The main operating hours are weekdays, see table 2:

Table 2: RDC region 3: Government Administrative Buildings for CEBEG Project

| Government Administrative Buildings | Location | Region | Building Use | Main operating hours |
| --- | --- | --- | --- | --- |
| Regional Democratic Council | Vreed-en-Hoop, West Demerara | Region 3 (Essequibo Islands West Demerara) | Oversees Local governance and development (public administration and governance) | Weekdays 7:30am-4:30pm |

Source: Interviews and respective organizations report

The main stakeholders, staff, mostly females, see table 3 below:

Table 3: RDC Region 3 Social Baseline Indicators

| Stakeholders | Gender composition RDC West Demerara |
| --- | --- |
| Staff | 106 of which 73 are females and 33 males |
| Onsite women | Est. population 73 |
| Customers | _ |

Source: Interviews with stakeholders of the institution

### Guyana Revenue Authority (GRA) – Parika

The Guyana Revenue Authority (GRA) Parika Office is in Parika Village, 50m from the Demerara River in Region 3 (Essequibo Islands- West Demerara). It is an administrative building that belongs to the Government of Guyana. The date of construction is unknown with renovations made in 2022. The total area of construction is 414 m2. It is used as an office building, though the second floor serves as the residence of the Manager.

In 2024, the total energy consumption was estimated at 54 477 kWh with an estimated cost of 3 204 882 GYD. When it comes to the CO2 emissions, the total was 36 282 kgCO2. If compliant with building codes, the energy consumption of the building is expected to reduce by 13 614 kWh with the new average consumption estimated at 40 863 kWh. Concurrently, CO2 emissions are also decreased by 9 062 kgCO2.

Currently, there are no renewable energy system installations in the building.

Figure 3: Frontal view of GRA - Parika

Figure 4: Location of GRA - Parika

Parika GRA is situated within a commercial/business area which includes markets, shops, small businesses, and informal vendors, making it a center of daily economic and social activity. Residents, traders, and travellers utilise the Parika Stelling, on the west bank of the Essequibo River in Region 3. It serves as the primary ferry terminal connecting Parika to Bartica, Essequibo, and other riverine communities, facilitating the movement of passengers, goods, and vehicles across the region. Its strategic location along the main access road ensures easy connectivity with nearby communities and services, while also supporting regional trade, commerce, and social interactions. The main operating hours are noted in table 4:

Table 4: Parika GRA Government Administrative Buildings for CEBEG Project

| Government Administrative Buildings | Location | Region | Building Use | Main operating hours |
| --- | --- | --- | --- | --- |
| Guyana Revenue Authority | Parika | Region 3 (Essequibo Islands West Demerara) | Office (tax administration and vehicle inspection) and residence | Weekdays 8am-430pm |

Source: Interviews with organizations

The main stakeholders’ staff are three quarters female, see table 5:

Table 5: Parika GRA Government Administrative Buildings Social Baseline Indicators

| Stakeholders | Gender composition GRA Parika |
| --- | --- |
| Staff | 22, of which 7 are females, and 15 males |
| Onsite women | Est. population 7 |
| Customers | _ |

Source: Interviews with respective institutions

### Guyana Elections Commission (GECOM) – Head Office

The Guyana Elections Commission (GECOM) Head Office is a complex of several buildings located on the corner of High and Cowan Streets in Georgetown – Region 4 (Demerara-Mahaica). Each building is organised in different departments. The main buildings are offices, the bond and the annex. The rest are auxiliary buildings like security rooms or parking spaces with low occupancy and low density of equipment. The date of construction is around the 1970s with a total area of construction of 414 m2.

In 2024, the total energy consumption was estimated at 95 659 kWh with an estimated cost of 5 099 208 GYD. When it comes to the CO2 emissions, the total was 63 908 kgCO2. If compliant with building codes, the energy consumption of the building is expected to reduce by 24 690 kWh with the new average consumption estimated at 71 268 kWh. Concurrently, CO2 emissions are also decreased by 16 444 kgCO2.

Currently, there are no renewable energy installations in the buildings.

Figure 5: Frontal view of main building of GECOM

Figure 6: Location of GECOM

The area surrounding the GECOM Head Office is within a dense urban environment characterized by a mix of commercial, administrative/government, residential and recreational services situated along a main access road. It is located near the Atlantic Ocean and hosts two major hotels that accommodate both visitors and business travellers. Weekday the building are open mainly as follows:

Table 6: GECOM Building

| Government Administrative Buildings | Location | Region | Building Use | Main operating hours |
| --- | --- | --- | --- | --- |
| Guyana Elections Commission Head Office | Kingston | Region 4 (Demerara Mahaica) | Elections administration and registration | Weekdays 8am – 4:30pm |

Source: Interviews with organizations

GECOM’s staff as the main stakeholder is predominantly females, see table 7:

Table 7: GECOM Buildings Social Baseline Indicators

| Stakeholders | Gender composition GECOM HQ |
| --- | --- |
| Staff | 147 of which 77 are females, and 70 males |
| Onsite women | Est. population 77 |
| Customers | _ |

Source: Interviews with respective institutions

Other social baseline components common to the government administrative buildings:

Livelihoods are indicated by employment categories and service improvement and/or disruption at the Government administrative buildings under consideration, see Figure 34 below. Disruptions are particularly important consideration for GRA Parika because of the very limited space it must accommodate customers and conduct vehicle inspections under. Usually when works become burdensome leading to limited vehicle inspections, longer customer waiting times, and even overtime pay requirement for staff to open the compound on weekends could become necessary. RDC Region 3 and GECOM HQ do not have similar space constraints but have older buildings for which safety considerations are compulsory.

Figure 34 Government Administrative Buildings Employment Categories

The Age range also varies among the government buildings’ organizations, for example the RDC Region 3 between 18-69 years old, and GECOM HQ between the ages of 18-72 years old, and GRA Parika data missing.

# Environmental and Social Management Plan

Identification and analysis of the environmental and social impacts were determined using the World Bank’s Environmental and Social Screening Form.

Table 10: Results of the Environmental and Social Screening of the government administrative buildings

| Questions | Answer |  | Comments |
| --- | --- | --- | --- |
|  | Yes | No |  |
| ESS1 - Assessment and Management of Environmental and Social Risks and Impacts |  |  |  |
| Is the subproject likely to have significant adverse environmental impacts that are sensitive and unprecedented that trigger the ‘Ineligible Activities’ and exclusion? |  | No |  |
| Does the subproject involve renovation or rehabilitation of any small-scale infrastructure, such as windows, doors, ceilings, or shelters? | Yes |  | The EE retrofits will incorporate energy-efficient windows, shading, wall and roof insulation, cool surfaces, etc. |
| Is the site close to a water body? | Yes |  | The GRA – Parika is in Parika which right on the eastern bank of the Essequibo River, near where it meets the Atlantic Ocean. The riverfront is roughly 100-150 metres with an immediate view of the river. |
| Will the subproject require clearing of vegetation? |  | No |  |
| Will the subproject be close to terrestrial/marine protected areas or sensitive area? |  | No |  |
| Will the subproject lead to sedimentation or soil erosion? |  | No |  |
| ESS2 - Labor and Working Conditions |  |  |  |
| Does the subproject involve uses of goods and equipment involving forced labor, child labor, or other harmful or exploitative forms of labor? |  | No |  |
| Does the subproject involve recruitment of workforce including direct, contracted, primary supply, and/or community workers? | Yes |  | The implementation of civil works and associated activities will require specific expertise and skilled labour. |
| Do workers need PPE relative to the potential risks and hazards associated with their work? | Yes |  | PPE is mandatory for all civil works and related activities to ensure compliance with occupational health and safety standards and to minimize the risk of accidents, injuries, and exposure to hazardous conditions. |
| Is there a risk that women may be underpaid when compared to men when working on the project construction? |  | No |  |
| Does the project lead to any risks and impacts on, individuals or groups who, because of their particular circumstances, may be disadvantaged or vulnerable.4 |  | No |  |
| ESS3 - Resource Efficiency and Pollution Prevention and Management |  |  |  |
| Is the project likely to generate solid or liquid waste that could adversely impact soils, vegetation, rivers, streams, or groundwater? |  | No |  |
| Do any of the construction works involve the removal of asbestos or other hazardous materials related to construction works? |  | No |  |
| Are works likely to cause negative impacts to air and/or water quality? | Yes |  | The works are likely to cause temporary negative impacts to air quality due to the nature which includes drilling, excavation, mounting of solar panels, and associated civil works. |
| Does the activity rely on existing infrastructure that is inadequate to prevent environmental impacts? |  | No |  |
| Does the activity require civil works to support project activities? | Yes |  | The DPV systems will require minor civil works, including structural preparation of rooftops, installation and anchoring of mounting systems, drilling for fastenings, cable management, and minimal site preparation to ensure safe and secure installation. |
| Does the activity require the use of pesticides, mildewcides or other chemicals? |  | No |  |
| Is there a potential that the works will involve sites that are affected by issues related to mold? |  | No |  |
| Could the construction activities be a nuisance to other activities at the site, such as from dust, noise, construction vehicles etc.? | Yes |  | Construction activities could cause temporary nuisance to normal working activities. |
| ESS4 - Community Health and Safety |  |  |  |
| Is an influx of workers, from outside the community, expected? | Yes |  | The implementation of civil works and associated activities will require specific expertise and skilled labour. |
| Could the construction activities be a nuisance to community members, such as dust, noise, traffic etc.? | Yes |  | Construction activities could cause temporary nuisance to community members due to dust, noise, and increased traffic, some buildings are located within or near residential communities. |
| Could the construction activities disrupt the primary use of the site? | Yes |  | Construction activities could temporarily disrupt the primary use of the office buildings, as access to certain areas such as rooftops, or common spaces may need to be restricted during installation. |
| ESS5 - Land Acquisition, Restrictions on Land Use and Involuntary Resettlement |  |  |  |
| Does the subproject involve involuntary land acquisition? |  | No |  |
| Does the subproject involve physical and/or economic displacement of people? |  | No |  |
| ESS8 - Cultural Heritage |  |  |  |
| Will the subproject involve any civil works that could involve demolition, renovations, or refurbishment to a historical or archaeological or culturally significant site or facility? |  | No |  |

After screening, a risk assessment was carried out for each site. The magnitude of each risk/impact was assessed against the criteria for probability and severity. The probability of each risk/impact was rated “rare” (least probable) to “almost certain” (most probable) while the severity of each risk/impact was rated from “negligible” to (least severe) to catastrophic” (most severe),

Table 11: Risk Assessment Matrix

| Probability of risk/impact | Severity of risk/impact |  |  |  |
| --- | --- | --- | --- | --- |
|  | Low | Moderate | Substantial | High |
| High | Moderate | Substantial | High | High |
| Substantial | Moderate | Substantial | Substantial | High |
| Moderate | Low | Moderate | Substantial | Substantial |
| Low | Low | Low | Moderate | Substantial |

Based upon these ratings, each risk/impact was assigned a rating of “low”, “moderate”, “substantial” or “high.”

Table 12: Identification of impacts on the government administrative buildings

| E&S Component/Standard | Activities under the Project | Potential Impacts/Risk Rating |  |  |
| --- | --- | --- | --- | --- |
|  |  | Environmental | Social | Risk Rating |
| Infrastructure Upgrade (ESS 1,3) | Installation of energy efficiency measures like an external insulation layer on the roof, replacements of windows in conditioned rooms, ceiling mounted fans in offices and conditioned rooms, replacement of fluorescent tubes with LED lighting, installation of occupancy detection sensors in bathrooms and smart switches to control the electricity consumption in appliances and nighttime air conditioning shutdown Installation of distributed PV systems on roofs like solar panels | These works will enhance the building’s thermal comfort, energy efficiency, and electrical systems, thereby contributing to an overall infrastructure upgrade. Similarly, the installation of the DPV system will improve the building’s capacity to support renewable energy systems, thereby strengthening and modernizing the existing infrastructure without major reconstruction. | Minor disruption to normal business operations/commercial activities during installation works (noise, restricted access to offices), and potential safety risks to workers (falls from heights, electrical hazards). | Moderate |
| Air Quality (ESS 3) | Roof work (drilling, cutting, anchoring) Removal of windows, lighting, or insulation Movement of packaging materials, insulation, and metal off-cuts Trucks, forklifts, or carrying materials over unpaved areas | Dust and particulates released into the air can temporarily decrease local air quality, especially in and around offices or other occupied areas. | Inhalation of airborne particles may pose health risks to workers and building occupants (respiratory irritation, allergies, or exacerbation of asthma). | Moderate |
| Water Quality (ESS 3) | Exposure/release of dust, debris, waste materials, small residues | Minor contamination from construction residues (dust, debris, or other materials) washed off during rain events, could enter drainage systems or surrounding soil and temporarily affect water quality. | Possible minor health or hygiene risks to nearby users if contaminated runoff reaches areas used by staff, patrons or nearby residents and businesses | Low |
| Noise (ESS 3) | Noise from power tools, hammering, and metal/concrete drilling Minor noise from drilling, screwing, or mounting fixtures from the installation of fans, lighting, and sensors Noise from cutting or adjusting mounting structures, fastening panels, and using hand or power tools during PV panel installation and wiring Noise generated from demolition, dismantling, and handling materials, removal of old fixtures (windows, lighting, insulation) Noise from movement of construction materials, pallets, and tools, and from forklifts or vehicles if used nearby, material handling and transport on-site Occasional noise from electrical tools, testing equipment, or temporary generators (if used). | Temporary increase in noise levels from drilling, cutting, anchoring, and equipment installation (fans, insulation, PV panels, inverters), which may disrupt the surrounding acoustic environment. | Disturbance to employees, community members, street vendors, patrons and businesses due to elevated noise during working hours, which may interrupt commercial activities and cause discomfort or stress if not properly scheduled or controlled. | Moderate |
| Soil (ESS 1) | Temporary storage or staging of materials and equipment like pallets, insulation bundles, solar panels, or tools Delivery and movement of heavy construction vehicles or equipment such as trucks or forklifts moving around the site Spills of construction materials like adhesives, sealants, paint, or small amounts of metal off-cuts falling Dust, debris, or residues from rooftop or indoor work that are washed onto ground-level areas | Temporary placement of materials, equipment, or scaffolding on ground-level areas may slightly compact the soil. Drilling, cutting, or anchoring may produce dust or small residues that settle on nearby soil. Small amounts of adhesives, sealants, paint, or metallic off-cuts could contaminate the soil if spilled or improperly disposed. | Health and safety risks if debris or spilled materials create tripping hazards. Temporary inconvenience of outdoor areas normally used by staff, patrons, community members and street vendors are restricted during works. | Low |
| Land Use (ESS 1) | Temporary placement of construction materials or equipment Vehicle and equipment movement on site Minor ground-level demolition or removal of old fixtures Temporary containment or runoff management | Placement of materials, scaffolding, or equipment may temporarily restrict use of walkways, or parking areas. Trucks or forklifts moving on site may cause minor soil compaction or temporary disturbance of surfaces. Small areas cordoned off for fixture removal may temporarily limit access to offices. Temporary barriers or containment may occupy small ground areas. | Community members, businesses, street vendors, patrons and staff may need to use alternative areas for recreation, movement, or parking. Areas occupied by materials or vehicles may create minor hazards if not properly cordoned. Temporary changes in land use can slightly disrupt business operations/commercial activities or outdoor activities. | Moderate |
| Solid Waste (ESS 1) | Removed windows, lighting, insulation, packaging waste (e.g., cardboard, pallets, fasteners) and possible metal off-cuts. Periodic e-waste from sensors, lights, and switches. | Generation of waste from removed windows, lighting fixtures, insulation, packaging materials (cardboard, pallets, fasteners), and metal off-cuts, may increase solid waste volumes, if not properly managed or disposed. | Improperly managed waste could create safety hazards for staff, patrons and workers, and cause nuisance or minor health risks. Accumulation of waste may affect cleanliness, hygiene, and visual appeal, potentially causing discomfort. | Moderate |
| Hazardous Waste (ESS 3) | Removal and replacement of old fixtures like windows, doors, lighting fixtures, and insulation Generation of waste like packaging, shipping materials, metal off-cuts and small construction debris Replacement of electrical or electronic components like old switches and small electronic parts General housekeeping and cleaning during installation can create dust, sweepings, and minor scrap materials | Disposal of small quantities of hazardous waste, such as fluorescent and mercury vapor bulbs, could release mercury or other toxic substances, potentially contaminating soil, water, or air if not handled correctly. | Improper handling or disposal could pose health risks to workers, patrons and staff, including exposure to mercury or other toxic substances. | Moderate |
| Biodiversity / Natural Habitat (ESS 1,3) | No vegetation removal or habitat disturbance as all work is within existing structure and rooftop system avoids intrusion on natural areas. | No impact on biodiversity or natural habitats is expected, as all activities are confined to the existing building structure and rooftops, with no vegetation removal or intrusion into surrounding natural areas. | No direct social impacts are anticipated related to biodiversity or habitat, since local ecosystems are not affected by the works. | Low |
| Labour and Workforce (ESS 2) | Interaction with contracted staff/project workers and customers require professional conduct, Following worksite code of conduct, labour laws and regulations, Hiring of project staff to work onsite Worker’s awareness of Anti-discrimination and SEA/SH, and GRM |  | Labour employed by contractors may be required to interact with onsite staff/project staff/ utilizing close spaces. Risk exists, especially when contractors employ non-English-speaking workers who find it difficult to communicate and follow local labour regulation and worksite code of conduct. Potentially too operating in space that customers utilise. | Moderate to Substantial at Parika GRA Moderate at GECOM HQ and RDC Region 3 |
| Community and Site Use ESS 4 | Construction and installation works Material storage and equipment placement Noise- and dust-generating activities Vehicle movement and deliveries Waste management activities Engagement with staff, patrons, street vendors, businesses and local community | Construction activities (drilling, cutting, anchoring) may temporarily increase noise levels, affecting indoor and outdoor acoustic conditions. Movement of materials, debris, and equipment can generate dust, affecting air quality in and around offices. Storage of materials, scaffolding, and equipment may cover walkways, or parking areas, limiting access and minor surface disturbance. Temporary storage of construction debris may affect site cleanliness if not properly managed. | Roof insulation, PV panel mounting, fan and lighting installation, wiring, and sensor setup may temporarily restrict access to certain areas of the buildings. Temporary placement of scaffolding, tools, and materials may occupy walkways, corridors, or parking areas, affecting normal use. Trucks and forklifts bringing materials may temporarily restrict access to building entrances or roads. Nearby residents, businesses, street vendors may experience temporary noise, dust, or minor traffic disruptions due to material deliveries or construction activities. Lack of prior notice may cause frustration or misunderstandings among staff, vendors, or neighbours. | Substantial |
| Cultural and Historical Sites ESS 8 | No project activities are expected to affect cultural or historical heritage, as all works are confined to existing building structures and rooftops. | No environmental impacts are expected, as the project activities are not located near any known cultural or historical sites. | No social impacts related to cultural or historical heritage are anticipated. | Low |
| Health and Safety ESS 2,3 | Roof work (drilling, cutting, anchoring, panel mounting) Electrical installation (wiring, inverter setup, lighting, sensors) Handling and lifting of heavy materials and equipment Removal of old fixtures (lighting, insulation, windows) Dust- and noise-generating activities Site movement and logistics | Minor dust generation as debris may settle on surrounding surfaces. Potential for minor spills of lubricants or cleaning fluids if used. Temporary movement on outdoor surfaces may slightly disturb soil. Temporary air quality and acoustic impacts; no long-term environmental damage. Minor disturbance to outdoor surfaces; no permanent land use change. | Risk of falls from heights Risk of injury from tools or falling objects Risk of electric shocks, burns, or short circuits Risk of fire if not handled properly Risk of strains and sprains Risk of accidents from improper lifting or carrying Risk of cuts, abrasions from broken materials Risk of respiratory issues from dust inhalation Hearing risks from prolonged exposure to loud noise Risk of slips, trips, and falls Risk of vehicle or equipment collisions | Substantial |

Table 13: Mitigation and Monitoring Arrangements for Government Administrative Buildings

| Anticipated Risks and Impacts | Proposed Risk Mitigation Measures | Impact Mitigation |  |
| --- | --- | --- | --- |
|  |  | Timing/Frequency | Responsibility |
| AIR QUALITY |  |  |  |
| Health of staff, and patrons, air quality, and building comfort may be potentially affected by emission of dust particulates from roof work, drilling, and emissions from heavy vehicles | Wetting of work surfaces (e.g. roofs, walls, drilling areas) | Construction phase | Contractor – Site Supervisor |
|  | Use dust barriers and tarpaulins especially near offices and parking lots | Before construction phase commences | Contractor |
|  | Schedule work during low occupancy periods e.g. after work hours, weekends, holidays | Planning phase and daily activity planning in construction phase | Contractor – Project Manager/Building Manager |
|  | Routine cleaning of site and walkways to remove settled dust | Throughout construction phase | Contractor – Site Supervisor |
|  | Temporary relocation of vulnerable groups like older persons | As needed, if heavy dust is generated and sensitive groups are identified | Building Manager |
|  | Communication of activities to staff, community members, businesses and street vendors | Planning and construction phases | PIU, Contractor, Building Manager |
|  | Monitoring of particulate matter during peak activities near offices | Weekly | Contractor - ESHS Expert |
| WATER QUALITY |  |  |  |
| Health, building cleanliness and local drainage systems may be affected during rain events, as construction residues (e.g., dust, debris, paint chips, insulation fragments, packaging materials) may wash off into drains, surrounding grounds, or watercourses. | Cover stockpiles and materials with tarps | Before construction commences | Contractor - Site Supervisor |
|  | Install barriers around the perimeter of work areas | Before any roof work is done during the construction phase | Contractor |
|  | Minimize exposed areas by completing sections in phases | Planning phase | Contractor |
|  | Sweep work areas regularly to remove dust and light debris | Throughout construction phase | Contractor – Site Maintenance/ Building Manager |
|  | Post rain inspection of work zones and drains for any signs of contamination | After any rainfall period during construction phase | Contractor – ESHS Expert |
|  | Temporary bins with lids for packaging and materials | During construction phase | Contractor – ESHS Expert |
|  | Monitor water quality after rain or weekly downstream of runoff area | During construction phase | Contractor – ESHS Expert |
| NOISE |  |  |  |
| Disruptions to business operations and market activities, discomfort to staff, community members, street vendors, patrons or health risks with prolonged exposure from temporary increase in noise levels due to construction activities such as installation of fans, insulation, solar panels, drilling, anchoring, and inverter setup. | Schedule noisy activities outside working hours (early mornings, weekends, holidays) | Planning phase and construction planning and daily scheduling | Contractor/ Building Manager |
|  | Notify staff, street vendors, community members, patrons in advance about planned noisy activities | 24-48 hours prior to activities | Contractor/ Building Manager |
|  | Use low-noise equipment and tools where feasible | Planning and construction phases | Contractor |
|  | Relocate sensitive activities like meetings from noise | Planning and construction phases | Contractor/Building Manager |
|  | Limit duration of high-noise tasks per zone | Planning phase | Contractor - Site Manager |
|  | Worker awareness training on noise protocols and staff sensitivity | Planning phase | Contractor – ESHS Expert |
|  | Maintain equipment to avoid excessive noise releases | Weekly during construction | Contractor |
|  | Monitor noise levels (dB) outside offices near construction | Daily and/or during peak activity | Contractor – ESHS Expert |
| SOLID WASTE |  |  |  |
| Environmental pollution, safety hazards, and unpleasant sights from solid waste generated during construction and retrofitting activities, including old windows, lighting fixtures, insulation material, packaging (plastic, foam, cardboard), metal cut-offs, and eventual electronic waste (inverters, batteries, panels). | Handle removed windows and glass frames with care, segregate from general waste, reuse frames if possible or store for safe disposal by NDC approved contractor | Construction phase | Contractor, NDC |
|  | Separate fluorescent from Hg bulbs and temporarily store in labelled boxes, avoid breakage until final disposal by certified hazardous waste handler / EPA-approved disposal facility | Construction phase | Contractor, NDC |
|  | Flatten cardboards, reduce single use packaging before final disposal by approved NDC contractor | Construction phase | Contractor, NDC |
|  | Remove sharp edges of any mounting brackets | Construction phase | Contractor |
|  | Store e-waste in dry secure area until disposal is arranged by certified hazardous waste handler / EPA-approved disposal facility | During operation and maintenance | Contractor/EPA |
| Improper handling/ disposal of FL bulbs can lead to mercury contamination of air, soil, and water which may risk staff health through direct exposure or accidental breakage. | Careful removal of FL bulbs to avoid breakage | As FL bulbs are removed during construction phase | Contractor - Electrical Technician |
|  | Use of sealed, labelled storage containers for used bulbs (e.g., “Hazardous - Used FL Bulbs”) in dry, ventilated area away from public access in secured storage areas within the building until final disposal by certified hazardous waste handler / EPA-approved disposal facility | Immediately after removal during construction phase/ Daily visual check during construction phase | Contractor – Site Supervisor |
|  | Separate hazardous waste stream from general construction waste | Throughout construction phase | Contractor – ESHS Expert |
|  | No on-site crushing or breaking of bulbs | Continuous through construction phase | Contractor – ESHS Expert |
|  | Train workers on handling, spill response and use of PPE | One time, refresh as needed during construction phase | Contractor – ESHS Expert |
|  | Coordinate with certified hazardous waste handler / EPA-approved disposal facility - Notify the EPA Guyana of the volume and type of hazardous waste generated. - Engage an EPA-approved contractor or recycler for final disposal. - Keep a register of hazardous waste movement for auditing purposes. | During the planning and construction phases | Contractor/ Building Manager, EPA, NDC and approved Contractor for waste disposal |
|  | Monitor the number of FL bulbs removed as it occurs, the storage condition and access weekly and any incidents of bulb breakage immediately | Planning and construction phases | Contractor – ESHS Expert |
| TRAFFIC/VEHICULAR MOVEMENT |  |  |  |
| Disruption to traffic within and around the building during material deliveries. Safety risks to staff due to movement of trucks and construction vehicles. Temporary obstruction of building access or parking areas. | Schedule material deliveries outside of peak office hours to minimize traffic disruption. Notify nearby residents, businesses, and road users in advance about the expected increase in construction and heavy vehicle traffic to minimize inconvenience and ensure public safety. Use designated areas within the premises, such as service yards or parking lots, for temporary parking of trucks and storage of materials. Coordinate with building management to communicate delivery times and access routes. Ensure vehicles follow safe speed limits within business grounds and implement traffic marshals if necessary. | During planning and construction phases | Contractor/Building Manager |
| Improper or unsafe storage of construction materials on-site. | Clearly mark and cordon off material storage areas to ensure safe movement of staff and patrons | During planning and construction phases | Contractor/Building Manager |
| SOCIAL |  |  |  |
| Disruptions | Schedule work during holidays or weekends | Construction phase /daily | Contractor/Building Manager |
|  | Communicate benefits to staff, community members, street vendors, patrons | Operation phase/monthly | Contractor and PIU |
|  | Awareness and enforce code of conduct and safety | Prior to construction/daily | Contractor |
| Health and safety | Signage, no access zone for staff and patrons, awareness for staff and | Construction phase/daily | Contractor/Building Manager |
| Gender Inclusion | Ensure safety in common areas; consult with female staff members | Design and installation phases/weekly | Human Resources Department /PIU |
| Labour issues (Working conditions, code of conduct, SEA/SH, and anti-discrimination policies) | Contracted employees should sign contracts that are clear on LMP policies including workers’ code of conduct, conditions, SEA/SH, and awareness of workers’ GRM | Implementation phase /monthly and immediately once there is a change in staff | Contractor and PIU |

## Implementation Arrangements and Reporting

Project Implementing Unit (PIU)

An established Project Implementing Unit (PIU) within the Guyana Energy Agency (GEA) will be responsible for the implementation and management of the activities under the CEGEB Project. The Project Manager will report to the Chief Executive Office of the GEA.  Roles of the PIU include but are not limited to:

- Raising awareness about the Project;

- Planning and managing the implementation of the Project;

- Subprojects screening to ensure eligibility criteria are met and developing the subproject pipeline;

- Supervision, reviewal and approval of all energy audits, designs and renovations;

- Conducting the procurement of country specific activities;

- Participating in the pooled procurement activities led by the OECSC PIU

- Project financial management;

- Compliance with environmental and social requirements, and fiduciary requirements;

- Ensuring coordination between the National Project Implementation Unit and the Regional Project Implementation Unit;

- Project monitoring and reporting to the World Bank and the Ministry of Finance, Guyana

The Environmental and Social (E&S) aspects will be managed by the Environmental and Social Officer. As part of the PIU, they will be responsible for ensuring the implementation of each investment according to the site-specific documents developed, such as ESMPs, OHSPs, and sub-project management plans. The E&S Officer will monitor the correct implementation of the E&S measures as part of periodic project monitoring visits while works are taking place at the sub-project site. The PIU will comprise existing staff from the GEA and new hires as needed by the Project, which will include the following roles:

Figure 9: Organisational Structure of the Project Implementing Unit

KEY

| Project Manager (PM) | Environmental and Social Officer (ESO) |
| --- | --- |
| Energy Engineers (EEn) x 2 | Public Communications Officer (PCO) |
| Electricians (E) x 3 | Procurement Officer (PO) |
| Monitoring and Evaluation Officer (MEO) | Financial Management Specialist (FMS) |

Contractor

- The Contractor will comply with all the project’s E&S risk management plans and procedures, which are integrated into the bidding documents, including ESMPs, Environmental and Social Consultation Plans, Labour Management Procedures, sub-management plans and national legislation. The contractor will ensure that construction-related grievances are received and addressed to the Grievance coordinator and resolve the grievances that could be resolved on-site. The contractor immediately notifies the E&S Officer about any significant incident (accidents, spills, fatalities, etc.) that may have significant adverse effects on the environment, community, the public or workers.

- The Supervision and Commissioning Consultant will oversee the daily implementation and monitoring of environmental, social and OHS mitigation measures, and report the progress of the sub-projects to the Guyana PIU E&S Officer through Monthly Monitoring Reports.

- The contractor ESHS Expert will provide input to the S&C Consultant for the preparation of the Monthly Monitoring Report on the ESHS performance. They will also disseminate and create awareness within the workforce regarding E&S risk management. The ESHS Expert ensures the stakeholders are informed and consulted before the commencement of any construction works and ensures continuous stakeholder engagement throughout the life of the Project. The contractor ESHS Expert prepares method statements for implementing E&S mitigation instruments and monitors site activities on a regular basis as defined in ESMP in compliance with E&S risk management instruments prepared for sub-projects.

Occupational Health and Safety and Governance

Occupational Health and safety concerns being addressed are important to mitigating the potential for time and other losses from implementing the Project. Occupancy schedules (in terms of main operating hours and the number of onsite users) are crucial discernment to comprehend and respond to EEMs. This guides the training and orientation needed for operations and sustainability of the installed capacity by the building manager.

Maintenance manager / HR head Stakeholders

The roles and responsibilities include:

- Support in communicating schedules, restrictions, and safety measures to staff.

- Participate in orientation sessions and grievance redress mechanisms.

- Report concerns or grievances to the contractor or PIU for resolution.

Reporting Framework

| Daily | Contractor’s ESHS Expert maintains site logs (incidents, waste management, PPE use, grievances received). |
| --- | --- |
| Weekly | Toolbox talks and short updates on compliance (by Contractor). |
| Monthly | Contractor submits a detailed ESMP compliance report to the Supervising Engineer/PIU. |
|  | Supervising Engineer consolidates monitoring findings and submits to the PIU. |
| Semi-Annual | PIU prepares E&S performance reports for the funding agency and regulatory authorities. |
| Ad hoc / Immediate | Incident reports (accidents, environmental spills, community complaints) must be submitted within 24 hours. |

## Institutional Strengthening and Capacity Building

Institutional strengthening and staffing are important to implementation as it ensures that appropriate skills and systems are in place to manage risks, comply with regulations, and sustain the benefits of the project. Considering this, the following aspects are proposed for consideration:

- Maintenance manager/building manager is required for management of the DPV system and the EEMs and any new AC systems. This can be facilitated through skill upgrade of an existing staff or hiring of a technician.

- Partnership with EPA is essential for the support of hazardous waste management and disposal.

- Environmental, Social, Health, and Safety appointment from the Contractor is recommended to ensure compliance and safe work zones.

Capacity development and training allow key stakeholders to identify, mitigate, and monitor environmental and social impacts. It promotes safe and compliant handling of hazardous and non-hazardous waste and ensures all parties understand their roles in safeguarding staff, community and patrons’ welfare during project execution. It also maintains longevity as institutional capacity is maintained for the energy systems after installation.

Table 14: Training Plan

| Training Topic / Activity | Target Group | Provider (Responsibility) | Timing / Frequency | Number of Trainings |
| --- | --- | --- | --- | --- |
| Environmental and Social Management | Contractor’s workforce | Contractor’s ESHS Expert | Initial (before works commence); refresher monthly or as needed | 4-6 |
| Occupational Health and Safety (OHS) | All site workers (including subcontractors) | Contractor’s ESHS Expert; Supervision Engineer (oversight) | Initial induction before mobilization; monthly toolbox talks; refresher quarterly | Ongoing (toolbox talks); 2 formal sessions |
| PPE Use and Safe Rooftop Work | Installation teams, site workers | Contractor’s ESHS Expert | Initial (before rooftop works); refresher mid-project | 2 |
| Hazardous Materials and Waste Handling | Relevant workers (handling wiring, batteries, lubricants, waste) | Contractor’s ESHS Expert | Initial training; refresher mid-project | 2 |
| Traffic and Vehicle Management | Drivers, logistics staff, site workers | Contractor | Initial; refresher if incidents occur | 1–2 |
| Code of Conduct Orientation | All contractor and subcontractor workers | Contractor’s HR Officer | Initial (before engagement); refresher monthly on renewal of contracts | 2 |
| Gender Sensitivity | All site workers; Contractor’s staff | Contractor’s HR Officer | Initial (before works start); refresher mid-project | 2 |
| Awareness for stakeholders | Staff, project workers | Contractor with support from Project Implementing Unit and staff from relevant institution | Before commencement of works; follow-up mid-project | 2 |

## Grievance Redress Mechanism

A Grievance Redress Mechanism (GRM) should be in place to allow affected stakeholders (including for workers) to voice concerns and seek timely resolution, based on risks including those unintended.

The critical steps involved in the GRM is noted in table 7 below, see annex 1 for forms:

Table 7: Compulsory steps for Grievance Redress Mechanism

| STEP | DESCRIPTION | ROLES AND RESPONSIBILITY |
| --- | --- | --- |
| SUBMISSION | Complaint submitted in writing or verbally to or by stakeholder | Affected stakeholder |
| RECORDING | Logged in a GRM register | Focal Point |
| ASSESSMENT | Assessed by PIU within 7 days. In cases of workers’ GRM assessed within 7 days as well | Grievance Redress Committee (GRC) |
| RESOLUTION | Provided within 10 days | GRC relate to Focal Point |
| APPEAL | If unresolved referred to local authority | Local Authority |

Complaints will be submitted to the PIU as the final or last resort and would be the responsibility of the Social and Environmental Officer. At each site the forms should be available via the building manager and/or respective HR. The phone number, email, and envelope (alternative to the drop box) should be available for those, especially workers, who are desirous of logging a complaint to the E&s officer of the PIU, but do not want to engage with the government administrative building authority on site.  They may also attend consultation meetings to raise concerns directly.

### Stakeholder Engagement

As noted in the ESMF 2024 the SEP defines a program for stakeholder engagement, including public information disclosure and consultations throughout the entire project cycle.

It is already recommended by the ESMF 2024 that start up consultations should “...discuss the Project’s objectives and activities, the grievance management process, and specific interventions planned for each site and potential impacts and risks related to the proposed project activities….”.

Stakeholder Identification and Analysis

For the CEGEB project the identification of core stakeholders, interests and potential concerns (see Table 10), at minimum, is compulsory for the monitoring and reporting process to meet the required standards of mitigating social risks and being responsive to stakeholders.

Table 15: Stakeholders’, Interests, and Needs

| STAKEHOLDER GROUP/AFFECTED PARTIES | INTEREST IN THE PROJECT | POTENTIAL CONCERNS / NEEDS |
| --- | --- | --- |
| STAFF | Improved WORKING environment; comfortable atmosphere for office | Safety during installation; noise disruption |
| WOMEN’S / VULNERABLE GROUPS | Equitable participation | Safety, participation in meetings or awareness platforms/tools |
| Office maintenance managers | Responsible for maintenance and operations of any additional capacity added to buildings | What EEM and DPV require to be maintained and its operation. |

Engagement Program

Site visits and virtual interactions have constituted stakeholder engagement thus far. Going forward

The method of engagement should expand in the following manner - focus group for general issues (either by sector, region, etc.), and key informant to be responsive to sensitive issues (example GBV) and vulnerable groups (example women). Bi-monthly meets should be sufficient, unless a need arises from the GRM which requires 7 days for review and 10 days for resolution, in an emergency.

Table 16: Stakeholder Consultation Engagement Plan

| Sub project Site type, name, and location | Stakeholder (s) | Method of engagement | Activities (including topic or theme covered) | Frequency | Responsible Party |
| --- | --- | --- | --- | --- | --- |

Monitoring and Evaluation of Engagement

As part of the stakeholders’ engagement process some core indicators must be captured to monitor and evaluate. The important characteristics of monitoring to be captured from stakeholders’ engagement are:

- Number of stakeholders consulted (Meeting attendance sheets per period)

- Diversity of participants (e.g., gender, age, designation, contact information – Participant records per period),

- Issues raised and addressed (Feedback log / GRM log)

Information Disclosure Plan

Information sharing is most important for disclosure, especially in compulsory awareness. The type of information, when, how, and in what format will vary. However, principally information can be shared for the CEBEG as follows:

- Project summary and timeline can be shared via flyers, notice boards, meetings and virtually

- Safety guidelines via posters, signage, building department heads, etc in an understandable language (graphics and creole English). GRM contact information are to be shared similarly.

It is important to consider the existing information sharing channels within the government administrative buildings under consideration, to avoid duplication and save resources.

### Cost

The costing table was developed by identifying all Environmental and Social (E&S) mitigation and monitoring measures proposed in the ESMP. For each ESMP activity (e.g. waste management, dust suppression, worker safety training, community engagement, PPE provision, SEA/SH awareness), the following were specified:

- Type and scope of the activity

- Frequency or duration of the activity

Integration into the Bidding Documents

These costs are not listed as separate provisional sums in the Bill of Quantities. Instead, they are integrated into the contractor’s unit rates for related construction activities, as required by good practice and procurement rules.

- The bidding documents will:

  - Include the ESMP as part of the tender package.

  - Clearly state that all costs to implement E&S mitigation and monitoring measures must be included in the contractor’s prices for the respective works.

This ensures that E&S compliance is mainstreamed into the project implementation, and that contractors allocate adequate resources (personnel, materials, and budget) to carry out the required mitigation and monitoring activities.

Table 17: Cost Estimates for all four institutions

| Activity / Measure | Frequency | Responsible Party | Costing Approach | Estimated Cost (USD) |
| --- | --- | --- | --- | --- |
| Provision of PPE (helmets, gloves, harnesses, safety boots, reflective vests) | Daily, throughout works | Contractor | Included in unit costs (site safety) | – |
| Toolbox talks / OHS refresher briefings | Weekly | Contractor | Included in contractor’s staffing cost | – |
| Formal OHS & Safeguards Training | Initial + refresher mid-project | Contractor | Included in contractor’s cost | – |
| Code of Conduct orientation | Initial (before works) + refresher annually | Contractor HR / Safeguard Specialist | Included in contractor’s mobilization costs | – |
| Waste collection and disposal (general + hazardous) | Weekly / as needed | Contractor | Included in site operations costs | – |
| Dust and noise control (e.g., wetting surfaces, noise barriers if needed) | Daily during works | Contractor | Included in construction methods | – |
| Traffic and vehicle management (scheduling, safe parking, marshals) | Daily during deliveries | Contractor | Included in logistics costs | – |
| Grievance redress mechanism (logbook, hotline, community liaison) | Continuous | Contractor / PMU Safeguard Officer | Included in contractor’s management cost | – |
| Orientation for staff, patrons, street vendors and members of the community | Twice (before works + mid-project) | Contractor/PIU/ | Small separate allocation | $2,000 |
| Contingency for unplanned E&S measures | As needed | Contractor | Separate allocation | $3,000 |
| Total Estimated E&S Budget | – | – | – | $7,000 |

# Annex

## Grievance Redress Forms

Table 18: Grievance Submission Form

| Date received: | Date: (dd-mm-yyyy) |  |
| --- | --- | --- |
| Grievant Name |  | □ Please keep my name confidential |
| Other Information: |  |  |
| Contact Information: | Phone: Email address: Address: (Please indicate the preferred method of communication) |  |
| Details of grievance: (Who, what, when, where) | □ One-time incident/complaint □ Happened more than once (indicate how many times): ___________ □ Ongoing (a currently existing problem) Details of the case: When did the case occur? Provide the location relating to the case/complaint. List the persons involved in the case. What are the effects of the ensuing situation? |  |
| How would you like to see issue resolved? |  |  |
| Attachments to the grievance: (e.g., pictures, reports etc.) | List here: |  |

_____________________________________	       ______________________________

Grievant Signature (if applicable)			       Date (DD/MM/YYYY)

_____________________________________                           ______________________________

Signature-Project personnel (to confirm receipt only)	Date (DD/MM/YYYY)

For PIU use only:

Grievance Category:

□ Power outages

□ Waste Management

□ Construction Quality/Equipment

□ Traffic/Road Access

□ Problem with staff (Project/Contractor/Sub-project site)

□ Consultations

□ Trainings

□Other (specify) ______________________________________________________

Grievance No: ______________________

Priority Level: Low/Medium/High________________________________________

Grievance Owner/ Department: ________________________________________

Table 19: Grievance Management Registration and Monitoring Sheet

| No. | Name of Grievant | Date Received | Grievance uptake channel used | Location/ sub-project site, if applicable | Gender of grievant/ | Grievance category | Grievance Description | Further Intervention? | Action(s) taken by PIU | Resolution Accepted/ Not Accepted Y/N | Date | Grievant Satisfaction score (1: Highly Unsatisfied-5: Highly Satisfied) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 20: Grievance Disclosure/Release Form (Result of Grievance Management)

| Grievance No: |  |
| --- | --- |
| Name of Grievant: |  |
| Date Filed: |  |
| Summary of Grievance: |  |
| Summary of Resolution: |  |
| Satisfaction Score of Grievant (1: highly unsatisfied – 5: highly satisfied) |  |
| Date of grievance resolution (DD/MM/YYYY): |  |

| Signature of Grievant in acceptance of the suggested grievance resolution: | Name: |
| --- | --- |
| ID type and number: | Date (DD/MM/YYYY): |

| Signature of Social Development Specialist | Signature of PIU Head: |
| --- | --- |
| Name: Place: Date (DD/MM/YYYY): | Name: Place: Date (DD/MM/YYYY): |